DUNKIN' LIGHTS - ESP32 16-CHANNEL (2x 8-RELAY) WIRING & PINOUT GUIDE

Printable Hardware Pinout & Wiring Checklist for All 10 ESP32 Units | 2026 Christmas Lighting Show (STEP_MS = 180ms)

1. RELAY MODULE #1 (CHANNELS 1 TO 8 -> LED STRIP SETS 1-8)

Wire IN1-IN8 of your FIRST 8-Channel Relay Module to these ESP32 screw terminals (P13-P32 on the Left row + P16 on the Right row):

| App CH | Relay Board Pin | ESP32 GPIO | Screw Terminal | Board Side | Assigned Light Group | Check |
| --- | --- | --- | --- | --- | --- | --- |
| CH 1 | Relay Module #1 - IN1 | GPIO 13 | P13 / D13 | LEFT Side | LED Strip Set 1 (Odd Group) | [ ] |
| CH 2 | Relay Module #1 - IN2 | GPIO 14 | P14 / D14 | LEFT Side | LED Strip Set 2 (Even Group) | [ ] |
| CH 3 | Relay Module #1 - IN3 | GPIO 27 | P27 / D27 | LEFT Side | LED Strip Set 3 (Odd Group) | [ ] |
| CH 4 | Relay Module #1 - IN4 | GPIO 26 | P26 / D26 | LEFT Side | LED Strip Set 4 (Even Group) | [ ] |
| CH 5 | Relay Module #1 - IN5 | GPIO 25 | P25 / D25 | LEFT Side | LED Strip Set 5 (Odd Group) | [ ] |
| CH 6 | Relay Module #1 - IN6 | GPIO 33 | P33 / D33 | LEFT Side | LED Strip Set 6 (Even Group) | [ ] |
| CH 7 | Relay Module #1 - IN7 | GPIO 32 | P32 / D32 | LEFT Side | LED Strip Set 7 (Odd Group) | [ ] |
| CH 8 | Relay Module #1 - IN8 | GPIO 16 | P16 / RX2 / D16 | RIGHT Side | LED Strip Set 8 (Even Group) | [ ] |

2. RELAY MODULE #2 (CHANNELS 9 TO 16 -> CHRISTMAS LIGHT SETS 1-8)

Wire IN1-IN8 of your SECOND 8-Channel Relay Module to these ESP32 screw terminals on the RIGHT row:

| App CH | Relay Board Pin | ESP32 GPIO | Screw Terminal | Board Side | Assigned Light Group | Check |
| --- | --- | --- | --- | --- | --- | --- |
| CH 9 | Relay Module #2 - IN1 | GPIO 17 | P17 / TX2 / D17 | RIGHT Side | Christmas Light Set 1 (Odd Group) | [ ] |
| CH 10 | Relay Module #2 - IN2 | GPIO 18 | P18 / D18 | RIGHT Side | Christmas Light Set 2 (Even Group) | [ ] |
| CH 11 | Relay Module #2 - IN3 | GPIO 19 | P19 / D19 | RIGHT Side | Christmas Light Set 3 (Odd Group) | [ ] |
| CH 12 | Relay Module #2 - IN4 | GPIO 21 | P21 / D21 | RIGHT Side | Christmas Light Set 4 (Even Group) | [ ] |
| CH 13 | Relay Module #2 - IN5 | GPIO 22 | P22 / D22 | RIGHT Side | Christmas Light Set 5 (Odd Group) | [ ] |
| CH 14 | Relay Module #2 - IN6 | GPIO 23 | P23 / D23 | RIGHT Side | Christmas Light Set 6 (Even Group) | [ ] |
| CH 15 | Relay Module #2 - IN7 | GPIO 4 | P4 / D4 | RIGHT Side | Christmas Light Set 7 (Odd Group) | [ ] |
| CH 16 | Relay Module #2 - IN8 | GPIO 5 | P5 / D5 | RIGHT Side | Christmas Light Set 8 (Even Group) | [ ] |

3. POWER & GROUND CONNECTIONS (CRITICAL FOR RELAY STABILITY)

| Line | Relay Board Terminal | ESP32 Pin | Screw Terminal | Function / Notes | Check |
| --- | --- | --- | --- | --- | --- |
| VCC | Relay Module #1 & #2 VCC | 5V / V5 / VIN | 5V or V5 Screw Terminal | 5V DC Power for Relay Optocouplers | [ ] |
| GND | Relay Module #1 & #2 GND | GND | GND Screw Terminal | Common Ground (Tie ESP32 GND + Relay GND) | [ ] |
| LED | Onboard Status Indicator | GPIO 2 | P2 / D2 (Built-in LED) | Solid ON = Wi-Fi Connected; Blinks = UNIT_ID | [ ] |

4. 10-UNIT HARDWARE SELF-ID REFERENCE TABLE (UNITS 1 TO 10)

All 10 ESP32 units share the exact same 16-pin relay wiring above. On boot or when you tap 'Hardware Self-ID' in the mobile app, each board blinks its onboard blue LED and clicks its matching relay channel:

| Unit Slot | Arduino Code (Line 24) | Blue LED Blink | Relay Channel Clicked on Self-ID | Flashed |
| --- | --- | --- | --- | --- |
| UNIT-01 | #define UNIT_ID 1 | Blinks 1x | Clicks CH 1 (GPIO 13 - Mod #1 IN1) | [ ] |
| UNIT-02 | #define UNIT_ID 2 | Blinks 2x | Clicks CH 2 (GPIO 14 - Mod #1 IN2) | [ ] |
| UNIT-03 | #define UNIT_ID 3 | Blinks 3x | Clicks CH 3 (GPIO 27 - Mod #1 IN3) | [ ] |
| UNIT-04 | #define UNIT_ID 4 | Blinks 4x | Clicks CH 4 (GPIO 26 - Mod #1 IN4) | [ ] |
| UNIT-05 | #define UNIT_ID 5 | Blinks 5x | Clicks CH 5 (GPIO 25 - Mod #1 IN5) | [ ] |
| UNIT-06 | #define UNIT_ID 6 | Blinks 6x | Clicks CH 6 (GPIO 33 - Mod #1 IN6) | [ ] |
| UNIT-07 | #define UNIT_ID 7 | Blinks 7x | Clicks CH 7 (GPIO 32 - Mod #1 IN7) | [ ] |
| UNIT-08 | #define UNIT_ID 8 | Blinks 8x | Clicks CH 8 (GPIO 16 - Mod #1 IN8) | [ ] |
| UNIT-09 | #define UNIT_ID 9 | Blinks 9x | Clicks CH 9 (GPIO 17 - Mod #2 IN1) | [ ] |
| UNIT-10 | #define UNIT_ID 10 | Blinks 10x | Clicks CH 10 (GPIO 18 - Mod #2 IN2) | [ ] |

5. IMPORTANT WIRING & BENCH TESTING NOTES

* Active-LOW Relays: Standard 8-channel optocoupler relay modules turn ON when GPIO is LOW (0V) and OFF when HIGH (3.3V). RELAY_ACTIVE_LOW = true is pre-configured in esp32_dunkin_lights.ino.

* Boot-Safe Pins: This pinout avoids ESP32 strapping pins GPIO 0, GPIO 12, and GPIO 15 so your ESP32 will never hang on boot or fail USB upload while the 2x 8-channel relay boards are connected.

* Dedicated 5V Power Recommended: All 16 relay coils active at once draw ~1.1A at 5V. Power the relay modules from a 5V (>=2A) supply and connect Relay GND to ESP32 GND.